PAWAN SHAKTI

Decentralized Wind Energy for the Inland World — Technical & Business Briefing Note

Pawan Shakti is developing a compact, low-cost small-scale wind turbine (SSWT) system engineered specifically for the low-wind conditions typical of inland India — where conventional wind and solar technologies are unreliable or unavailable. The system pairs CFD-optimized multi-propeller blades with a compact planetary gearbox to generate usable power at wind speeds as low as 1.5–3.5 m/s, well below the 5 m/s+ threshold required by conventional small-wind systems.

Mission:  “Apna ghar, apna turbine.”

Founder:  Yaswanth

Incubator:  VNIT Venture Vault Foundation (V3F)

Academic Mentor:  Dr. J. Bhukya

The Problem & Our Differentiation

Conventional small-wind systems require wind speeds above 5.0 m/s to begin generating power, leaving most inland Indian sites — where average wind speeds are far lower — unserved by wind energy. Solar alone is intermittent, and existing small-wind hardware is bulky, complex, and expensive to install.

| Vector | Conventional Small-Wind | Pawan Shakti |
| --- | --- | --- |
| Cut-in wind speed | > 5.0 m/s required | Efficient at 1.5–3.5 m/s (micro-wind ready) |
| Availability | Highly intermittent (solar/wind) | Maximum active time; bypasses solar gaps |
| Footprint | Bulky, industrial, unfit for rooftops | Compact, simple local assembly |
| Unit cost | Prohibitively high | ~₹35,000 for mass deployment |

Hero Product: 100W Modular Wind System

A 100 W modular SSWT explicitly optimized for wind speeds under 5 m/s, combining:

- CFD-designed multi-propeller blades for enhanced aerodynamic efficiency at low wind speeds

- A compact planetary gearbox-generator system (pressure angle –20°) for speed and torque enhancement

- A DC generator rated 110 V DC, 3500 RPM, 100 W per unit, feeding a pure sine-wave DC-to-AC inverter (230 V AC, 50 Hz) via a 12 V, 100 Ah battery

- Cut-in speed 2.3–2.5 m/s, cut-off speed 25 m/s — powering mobile charging, LED/CFL lighting, fans, small agricultural water pumps, and other domestic/agricultural loads

The system has progressed through Mark-3, Mark-4, and Mark-5 prototypes, with three- and four-propeller 100 W configurations validated in wind tunnel testing at VNIT Nagpur, and is supported by two academic papers: “Affordable Low-Wind Rooftop Wind Turbine Energy System Design for Rural Electrification” and “A Space-Efficient Multi-Rotor Wind Turbine System for Decentralized Power.”

IP & Strategic Backing

- Published patent: “Compact Wind Turbine with Planetary Gears” (2025); active filing in progress for turbine and magnetic gear integration

- Supported by ANRF accelerated research grants and the CBDE R&D scheme

- Incubated by VNIT Venture Vault Foundation (V3F)

Where This Fits

Pawan Shakti is positioned as a complement to rooftop solar rather than a replacement — providing decentralized, reliable power that offsets solar's intermittency for rural households, small farms, micro-grids, telecom towers, and remote monitoring stations. It targets segments across residential rooftops, rural schools & clinics, and disaster-relief backup power.

Our Aim

Our goal is to take Pawan Shakti beyond a validated lab prototype and build it into a real, industry-grade product — one that is manufacturable at scale, reliable in the field, and ready for wider deployment. This means strengthening the engineering (blade aerodynamics, gearbox and generator design, power electronics), running structured field trials to prove durability and energy yield under real-world conditions, and setting up a repeatable local-assembly and manufacturing process.

Beyond the turbine itself, this project is central to growing the startup as a whole — building out the team, formalizing our IP position, and establishing the partnerships and manufacturing know-how needed to move from a university project to a functioning company.

Core Team

Yaswanth (Founder & Lead) drives strategy, vision, and commercialization, with Dr. J. Bhukya as Academic Mentor providing strategic oversight and research guidance. The engineering team covers CAD & modeling, blade design (CFD/aerodynamics), electronics (embedded systems & PCB), system integration, generator R&D, power electronics, prototyping/workshop operations, and field deployment logistics.