Wind Turbine Data Collection — Intern Task Brief

Duration: 2 Days   |   Project: Yashvant Turbine — Smart Optimization System

# Goal

Collect a clean, well-labeled sensor dataset from all three turbine configurations, across a range of wind conditions, and use it to train a working baseline power-prediction model. By the end of Day 2, we need a spreadsheet of continuous, time-stamped readings — with wind speed included — plus a first-pass trained model with reported accuracy, ready to be improved with more data later.

# What We Already Have

- ESP32 gateway set up to log sensor data (to Excel/Sheet or via serial)

- Sensors: Voltage (V), Current (A), RPM, Vibration (X, Y, Z — if available), Wind Direction

- Three turbine configurations to test (Config A, B, C)

# What's New / Must Be Added

Wind speed is the most important missing parameter. Without it, we cannot fairly compare the three configurations — a config may look better simply because it was tested on a windier day. Wind speed must be logged at the same time and frequency as every other sensor reading.

- Add a cup anemometer (pulse/reed switch type) wired to an ESP32 interrupt pin — cheapest and easiest option.

- If a Davis/NRG anemometer is available, that also works (analog or pulse output).

- Log wind speed in the same row/timestamp as V, I, RPM, vibration, and direction — never as a separate slower log.

# Day 1 — Hardware Setup & Test Run

## Morning

☐  Wire up anemometer to ESP32 (interrupt pin for pulse counting)

☐  Write/update firmware to read wind speed alongside existing sensors

☐  Confirm all sensors report correctly together: V, I, RPM, Vibration X/Y/Z, Wind Speed, Wind Direction

## Afternoon

☐  Set up logging destination (Excel via Sheets webhook, or local CSV/SD card on ESP32)

☐  Create the column structure (see table below) in the logging sheet

☐  Run a 30–60 minute test log on ONE configuration to confirm data looks correct (no missing values, timestamps consistent, power calculates correctly)

☐  Fix any issues found during the test run before scaling up

# Day 2 — Data Collection + Baseline AI Model

## Morning — Continue Data Collection

☐  Start continuous logging on Configuration A

☐  Rotate through all 3 configurations across the day (e.g., A → B → C → A...) every 2–3 hours, so each config sees a similar mix of wind conditions — don't run one config only when it's windy and another only when it's calm

## Afternoon — Continue Rotation

☐  Continue rotation, aim for at least 2–3 hours of logged data per configuration

☐  Spot-check the sheet every hour: no gaps, no stuck sensor values, timestamps updating

## End of Day 2 — Clean Data + Train a Baseline Model

☐  Export/clean the final spreadsheet (remove test-run data from Day 1)

☐  Add a short notes column entry for anything unusual (e.g., 'gust,' 'sensor reset,' 'manual config change at 3:15pm')

☐  Load cleaned data into Python (pandas) — drop rows with missing/stuck sensor values

☐  Confirm Power = V × I is calculated; add simple extra features if time allows (e.g., rolling avg wind speed)

☐  Train a simple baseline model (Linear Regression, then Random Forest or XGBoost) to predict Power from V, I, RPM, Wind Speed, Wind Direction, Vibration, Config ID

☐  Do an 80/20 train/test split and report accuracy (R², MAE, or RMSE) — a working first pass, not the final model

☐  Save the trained model file (.pkl or .joblib) and the script used to train it

☐  Hand off: completed sheet, cleaning/training script, model file, accuracy numbers, and a 2–3 line summary of what was observed per configuration

# AI Training Scope (Day 2, Baseline Only)

This is a first-pass baseline model, not the full production system. The goal is to prove the pipeline works end-to-end — raw data → cleaned data → trained model → accuracy metric — using whatever is collected in this 2-day window. It gets retrained later once more data exists.

- Clean & preprocess: handle missing values, remove obvious sensor glitches (e.g., impossible RPM spikes).

- Feature engineering: Power (already calculated), Config ID as a categorical feature, optional rolling averages for wind speed/vibration.

- Model: start with Linear Regression, then try one tree-based model (Random Forest or XGBoost) and compare.

- Target variable: Power (W). Stretch goal if time allows: flag which Config ID gave highest power for similar wind speed.

- Report accuracy honestly — with only ~1 day of real data, don't expect high accuracy yet. The goal is a working pipeline, not a polished model.

# Required Data Columns

| Timestamp | Config ID | Voltage (V) | Current (A) | Power (W) | RPM | Wind Speed (m/s) | Wind Dir. | Vib X/Y/Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2026-07-29 09:00:01 | A | 12.4 | 2.1 | =V×I | 310 | 4.2 | NE | 0.02/0.01/0.03 |

Add a Notes column at the end for anything unusual observed during logging (gusts, manual changes, sensor resets).

# Rules for Good Data

- Log every reading with a real timestamp — never leave timestamp blank or estimated.

- Rotate configurations regularly (don't leave one config running all day) so wind conditions are shared fairly across all three.

- Don't edit/delete raw values after the fact — if something looks wrong, add a note instead of fixing the number.

- Keep sampling rate consistent (e.g., once every few seconds) — irregular gaps make the data harder to use.

# Deliverable (End of Day 2)

One spreadsheet (Excel or CSV) with continuous, rotated, timestamped data across all 3 configurations, wind speed included — plus a trained baseline model (script + model file + accuracy metrics) and a short written note on anything unusual observed.